Radiology

By: Andy Codding – January 2002

* Radiology on the National Board Exam

Obtaining and interpreting radiographs- approximately 45 questions

- Principles of radiophysics and biology – 4 questions

- Principles of radiologic health – 5 questions

- Technique, Errors – 6 questions

- Recognition of normal and abnormal – 5 questions

- Case Study questions – 30 Questions

  - Intraoral Anatomy – 3 questions

  - Panoramic Anatomy – 2 questions

  - Technique Errors – 7 questions

  - Tooth Conditions – 14 questions

  - Restorative Material – 4 questions

TIPS

- Be sure to check the labeling!  Sometimes the R & L are on different sides of the films than what we are used to seeing in school!

- When there is a question about amalgam tattoos, always double check with the radiograph.  If there was a recent extraction- it should tell you.

- Check x-rays for missing teeth, restorations, caries classifications

- When given an intra-oral picture, always refer back to the radiographs to be sure you are looking on the correct side and teeth numbers.   Intra-oral pictures can be a direct shot, or through a mirror.  Don’t get confused!

- Remember you can double check your Angles Classification with the bitewing radiographs.

Radiology Definition – The flow of energy through space and matter in the form of waves or particles.

### Ionizing radiation

Definition - Radiation that is capable of producing ions by removing or adding an electron to an atom.

- Characteristics –

    - Electromagnetic and particulate radiation can interact with atoms and create ion pairs.

    - Mechanism by which biological systems are altered or damaged

B) Examples

- Electromagnetic radiation

- Particulate radiation

Electromagnetic radiation –

Definition - Transmission of wave energy through space and matter as a combination of electric and magnetic fields.

##### Characteristics –

    - Have no mass and no electrical charge.

    - Travels at the speed of light in a straight but oscillating path (like ripples in a pond)

    - Electromagnetic spectrum is measured according to frequency, energy, and wavelength.

#### Examples

    - Long wavelengths – radar, television, radio

    - Short wavelengths- visible light, UV light, x-rays

# Particulate radiation-

Definition - Atomic nuclei or subatomic particles that travel at high velocity.

###### Characteristics –

    - Have mass and energy

    - All particles have an electrical charge (except neutrons)

    - Travel in straight lines at high speeds, but can not reach the speed of light

###### Examples

    - Alpha particles, beta particles, and cathode rays (electrons)

    - Nucleons – protons, neutrons

### Discovery of X-Radiation

      - Sir William Crookes -1878

        - Crookes tube, also known as “cathode ray tube”

        - Evacuated glass tube with + electrode at one end and – electrode at opposite end.

        - Various gases when injected would glow when electricity was applied.

- William Roentgen –1895

        - Discovered and termed “X-Rays”

        - Noticed finger bones glowing on phosphorescent paper

- Otto Walkoff – First dentist to attempt x-rays on patient (himself)

- W.J. Morton – NY physician made first dental radiograph in U.S. (a skull)

- Edmund Kells – First dental radiograph on a live patient in the U.S.

        - Father of dental radiology

        - Numerous cancers of his hands and arms.

### Electricity

Definition – The flow of electrons through a wire or other electrical conductor.

Types

- DC – Direct Current – electrons flowing in one direction

- AC – Alternating Current – electrons are constantly switching directions

## Units of measurement –

- Milliamperage (mA) – The quantity of the x-rays produced, regulates the temperature of the cathode filament

  - 1 mA= 1/1000 ampere

  - Typically fixed at a specific mA such as 10 mA

- Ampere – the unit of measurement used to describe the number of electrons passing through the cathode

- Volt – measures the energy level of electricity  (the force of electrons)

  - 1 kVp = 1000 volts

  - Usually fixed at 70 kVp

  - Variable kVp machines range from 50 – 100 kVp

- Timer – regulates the length of x-ray exposure, calibrated as fraction of a second

    - Formats

      - Standard fractions of a second (1/2 second)

      - Decimal fraction of a second (.50 second)

      - Impulses (30 impulses/second)

    - Conversion

      - To convert exposure time in seconds to impulses, multiply by 60 (1/2 X 60 = 30 impulses/second)

      - To convert impulses to exposure time in seconds, divide by 60 (30/60 = ½ or .50)

## Parts of the X-Ray unit

  - Generator- supplies electrical DC power to the x-ray tube

  - Transformer- Changes electrical supply to any desired level

- Step-down transformer – decreases electrical current from 110 or 220 volts to 8-12 volts to heat tungsten filament

- Step-up transformer – increases electrical current from 110 or 220 to 50 to 100 kVp to produce x-rays

- Autotransformer- allows selection of variable Kvp settings on x-ray units with a kVp range

  - Rectifier – The process of changing an alternating current into a direct current

- Half-wave rectifier- where half of the AC current is converted to DC

- Full-wave rectifier- where the entire AC current is converted to DC

  - Cathode (-) - Tungsten filament –thin wire (source of electrons)

  - Anode (+) - Tungsten target - copper rod which functions to conduct heat away from target

  - Lead diaphragm/collimator- device that restricts size of x-ray beam

  - PID – device that guides the x-ray beam toward patient

  - X-ray filter – layers of aluminum metal used to filter x-ray beam

A) 1.5 mm added filtration for machines operating at < 70kVp

B) 2.5 mm added filtration for machines operating at > 70 kVp

Steps to X-ray Production

- Tungsten filament is heated (controlled by mA settings via the step down transformer)

- Electrons are set in motion (+ anode attracts electrons from – cathode)

  - As electricity flows in the machine, the cathode filament is heated and free electrons are liberated from the filament called thermionic emission.

- Anode target bombardment  (<1% produce x-rays, >99% produce heat)

- Heat is conducted via the copper sheath/rod into the oil surrounding the x-ray tube

- X-rays leave the glass tube via the unleaded glass window

- Primary X-ray beam

  - Filtered

  - Collimated

  - Guided through the PID

## Anode Target Bombardment – 2 ways

Bremsstrahlung (Braking)

            - As bombarding electron from cathode passes near nucleus of target atom, its direction is changed and it loses speed.  Lost speed is transformed into x-rays

OR

            - Bombarding electron scores a direct hit on target atom nucleus, losing all of its motion.  Energy is converted to high energy x-rays

*Vast majority of x-rays are produced this way

Characteristic (Discrete)

- Bombarding electron from cathode strips electron rom target atom at the anode, knocking it out of orbit.

- An outer shell (higher energy) electron in the target atom drops down to take the empty spot.  The drop to the lower level causes a loss of energy in the form of x-rays

## Interaction of X-rays with Tissues

Secondary Radiation (Scatter) - Direction of primary radiation is altered by tissues, resulting in fogging of radiographic image

Primary Radiation + Interaction with Tissues = Secondary Radiation (Scatter)

Thompson Scattering (Coherent Scattering)

- X-ray photon is absorbed by tissue atom electron, but the electron is not displaced.

- No tissue ionization due to no electron lost

- Accounts for around 5% of primary radiation

- Has little effect on film fog

Photoelectric Effect

- X-ray photon dislodges inner-shell electron from tissue atom and ceases to exist

- Outer shell electron then drops into vacancy producing lower energy secondary radiation

- Tissue ionization due to loss of electron

- Accounts for 45% of primary radiation

- Energy is absorbed by the patient

- Has little effect on film fog

Compton Effect (Compton Scattering)

- X-ray photon strikes a blow to an outer shell tissue atom electron, dislodging it

- Deflected photon continues to exist as lower energy scatter radiation

- Tissue ionization due to loss of electron

- Accounts for 45% of primary radiation

- 30% of the scatter escapes the patient tissues

- Remainder of the scatter contributes to film fog

## Ionizing Effect on Cells

- Direct Effect- x-ray photon strikes critical structure (such as DNA)

- Indirect Effect- x-ray photon strikes a non-critical structure, the resulting scatter radiation will then strike a critical structure (rebounding)

- Somatic Effect –results in tissue injury to exposed individual (such as cancer or radiation burn)

- Genetic Effect – results in damage to gamete of exposed individual, yielding defects in the offspring

- Acute Effect- injury is immediately manifested, such as radiation burn or radiation sickness

- Chronic Effect- injury is manifested much later, such as developing cancer

## Dose-Response Relationships

  - Linear – tissue response correlates directly with dose

  - Non-Linear - tissue response correlates indirectly with dose

  - Threshold- below a certain radiation dose, there is no tissue response (radio-resistant tissues)

  - Non-Threshold- any dose results in some tissue response (radio-sensitive tissues)

## Sensitivity to Ionizing Radiation

Cells Most Sensitive:

- Rapidly dividing cells (high mitotic rate)

- DNA and chromosomes

- Undifferentiated cells (stem cells, unspecialized cells)

- Cells with high metabolic activity

Mnemonic:

High Education – Love Reading Books While Sippin’ Champagne

Moderate Education – Everyone Still Can’t Read Great

Low Education - Books Make Neglected Children Dumb - except Me!

The quality factor for diagnostic x-rays is “1”

1 Rad X 1= 1 Rem

## Units of Radiation Measurement

Traditional System

- Roentgen (R) – unit radiation intensity within a given space

- Rad (rad) – unit of tissue absorbed radiation dose

- Rem (rem) – absorbed dose based on type of radiation (quality factor)

- Curie (ci) – quantity of radioactive material which is emitting radiation

International System

- Coulomb (C ) – equal to 3880 roentgens

- Gray (Gy) – equals 100 rads

- Sievert (Sv) – equals 100 rems

- Becquerel (Bq) – similar as Curie

Intraoral radiographic film

- Outer plastic or paper cover – protects film from light and moisture

  - Plain white side directed to x-ray source (“white to the light”)

- Lead Foil – reduces film fog by absorbing secondary radiation scattering back from patients tissues

- Film base – tint reduces eye fatigue, provides stiffness with some flexibility

- Film emulsion- gelatin matrix with suspension of silver halide crystals

  - Silver halide crystals or grains

    - 95% silver bromide

    - Small amount of silver iodide

    - Sulfur containing, increases sensitivity of silver bromide crystals

- Adhesive – aids in adherence of emulsion to film base

- Supercoating – gelatin layer over emulsion to help prevent scratching.

## Extraoral radiographic film

- Intensifying screens- screens fluorescence upon exposure to x-rays

  - Reduces patient radiation dose

  - Shorter exposure time

  - Extended x-ray tube life

Basic components

- Base- plastic support material

- Reflecting layer- reflect light emitted from phosphor layer back to the film

- Phosphor layer- light sensitive phosphorescent crystals suspended in plastic material

  - Calcium tungstate screens – emit BLUE and blue-violet light

    - Must be matched with blue light sensitive film

    - Slower crystals that require more x-ray exposure

  - Rare earth screens – emit GREEN light

    - Must be matched with green light sensitive film

    - Faster crystals that require at least one half the x-ray exposure of calcium tungstate screens.

## Types of Extraoral Films

- Arthrography – joint radiography, good for TMJ

- Axial (basilar)(submentovertex) projection – “jug handle view”, cranial base

      - Caldwell projection – all aspects of the orbit, sinuses

      - Computed tomography (CT) – good for measuring tissue thickness when placing implants

      - Lateral (Cephalometric) head plate- common projection used for orthodontics to assess facial growth

      - Lateral Jaw- records right or left side of the jaws

      - Lateral Oblique mandibular projection- mandibular condyle, coronoid process, ramus and body of

Mandible

      - Lateral Skull projection – sinuses, nasopharynx, maxilla

      - Panoramic- records entire maxilla, mandible, and immediately adjacent structures, impacted teeth,etc

- Posterior-anterior (PA) projection- entire mandible except condyles

      - Towne & Reverse Towne projection- condyle, occipital bone, orbital floor, zygomatic arch, middle ear structures

- Waters projection- view useful for examining maxillary sinuses as well as frontal and ethmoid sinuses

## Dark Room

General Principles

    - Development- transforms latent image into visible image by reducing exposed silver halide crystals

      - Hydroquinome- (reducting agent) – brings up black tones and contrast

      - Elon- (reducting agent) brings up gray tones

      - Sodium cabonate (activator) – soften and swells the emulsion

      - Potassium bromide (restrainer) – prevents reducing agents from developing the unexposed silver halide crystals

      - Sodium sulfite (preservative) – prevents rapid oxidation

      - Water- (solvent) – medium for dissolving chemicals

    - Rinsing – stops development and removes excess chemicals from the emulsion

    - Fixation – removes unexposed silver halide crystals from the emulsion

1.Sodium or ammonium thiosulfate (clearing agent) removes unexposed silver    bromide crystals from the acidifier

- Acetic acid or sulfuric acid (acidifier) – lowers pH which enhances fixing agent

- Aluminum chloride or potassium alum (hardener) –shrinks and hardens the emulsion

- Sodium sulfite (preservative) – prevents oxidation of the fixer and prolongs its life

- Water – (vehicle) – medium for dissolving chemicals

D) Washing – removes all excess chemicals from the emulsion

E) Drying – removes the water from the emulsion

## Intraoral and Processing Errors

1. Vertical angulation errors – too much vertical angulation is used.

- Corrected by decreasing vertical angulation and placing film more parallel to the long axis of the tooth

2.  Elongation- not enough vertical angulation is used

- Corrected by increasing vertical angulation

3.  Horizontal angulation errors- overlapping

- Corrected by directing central ray perpendicular to facial surface of the teeth crowns

4.  Cone Cut errors- lack of centering x-ray beam over film

- Corrected by directing central ray to film center

5.  Overexposure- high density or dark films

- Corrected by reducing exposure time

6. Underexposure - low density or light film

- Corrected by increasing exposure time, make sure exposure button was not released too soon.

7.  Developer Stain – dark stains on film

8.  Fixer Stain – light stains on film

9.  Fog  - visible light that exposes film, unsafe darkroom illumination, safe light too close to working area

10.  Green Film – unprocessed film which has been opened and exposed to light

11.  Static Electricity - causes black lightening bolts or “naked tree” pattern

- Reticulation – causes cracks in emulsion that look like “cracked ice,” due to extreme temperature differences in processing chemicals and the wash water

13.  Fluoride Stain – causes dark staining on films

14.  Air Bubbles – cause light spots on film where develop failed to contact the emulsion

Panoramic Errors

1.  Smile image- patient chin is too low

2.  Frown image- patients chin is too high

3.  Blurred maxillary anterior teeth – patients chin is tilted upward

- Blurred mandibular anterior teeth – patients chin is tilted downward

- Cervical spine error- spinal column is slumped

6.  Radiolucent artifact is created in maxillary apical region – patient is NOT pressing tongue against palate

      - Density- overall degree of blackness on the x-ray

- Factors affecting density – exposure factors (mA, time, kVp) , source-film distance

  - May be increased by increasing exposure time

  - May be increased by increasing mA

  - May be increased by increasing kVp

      - Contrast- difference in densities among various regions on a x-ray

- Factors affecting contrast – kVp, subject contrast, film contrast

  - May be increased by decreasing kVp

      - Sharpness- ability of a radiograph to define an edge

- Factors affecting sharpness- focal spot size, movement, film emulsion grain size, intensifying screen fluorescence

D)     Resolution – ability of a radiograph to record and demonstrate separate structures that are close  .     .        together.

1. Factors affecting resolution – focal spot size, movement, film emulsion grain size, intensifying screen fluorescence

Buccal Object Rule

-SLOB Rule

- Same lingual opposite buccal

          - MBD Rule

          - If object moves in same direction as x-ray head, object is lingual in location

          - If object moves in opposite direction to x-ray head, object is buccal in location

Inverse Square Law

- The intensity of radiation is inversely proportional to the square of the distance from the source of radiation

- i1= d22                  I = Intensity

i2   d12                  D= Distance

Sample Problem:

If a person standing 3 feet from an x-ray source receives 4 Rads of exposure, how much would they receive at 6 feet?

I1= 4 Rads   D1=3’ D2=6’

X=   162  = .40 sec

82

If the distance is doubled = then you multiply by 4 --- If the distance is halved = then you divide by 4

Radiation Safety

Agencies

AlARA Principle- “As Low As Reasonably Achievable”

  - Methods to minimize patient exposure

    - Minimize exposure time, use fast film

    - Maintain long source-object distance

    - Utilize proper added filtration

    - Utilize appropriate lead shielding

    - Use rectangular vs. round collimation

Filtration

      - 1.5 mm added filtration for machines operating at < 70kVp

###### 2.5 mm added filtration for machines operating at > 70 kVp

      - All lead aprons should have at least .25mm lead thickness

      - Inherent filtration- unleaded glass window and oil bath that x-ray beam passes through

      - Added filtration- metal filter, usually aluminum, installed by manufacturer

Total Filtration = Sum of inherent and added filtration

Intraoral

          - Thyroid collar – 50% exposure reduction to thyroid

          - Cover front from neck to thighs

Extraoral

          - No Thyroid collar

          - Cover front and back from shoulders to waist

## MPD – Maximum Accumulated Dose

-Specific for job description

          - Assumes linear, non-threshold response

          - Is a cumulative measure of exposure

          - Limits for general public are 10% of limits for occupationally exposed workers

          - Formula implies that no one under age 18 may be occupationally exposed

Formula N=Age

MPD =  5 (N-18) Rem

Or

MPD = .05(N-18)Sv

MAD – Maximum Permissible Dose

- Accumulated lifetime dose limit for occupationally exposed workers

          - MAD for general public is 10% of limits for occupationally exposed workers

Formula N= Age

MAD = 5Rem

Or

.05Sv

Example:  MAD for a 35 year old radiation worker:  MAD = (35-18) X 5 = 85 rems or 850 mSV

Half Value Layer – Defines the thickness of material required to block 50% of x-rays

## Periapical Radiolucencies

- Periapical granuloma – a localized mass of chronically inflamed granulation tissue at the apex of a non-vital tooth.

- Periapical cysts (radicular cyst) – a lesion that develops over a prolonged period of time, cystic degeneration takes place within a periapical granuloma and results in a periapical cyst.

          - Results from pulpal death and necrosis

- Periapical abscess - a localized collection of pus in the periapical region of a tooth that results from pulpal death.

  - Acute periapical abcess- painful

  - Chronic periapical abcess- usually asymptomatic

## Periapical Radiopacities

- Condensing osteitis – a well-defined radiopacity that is seen below the apex of a non vital tooth with a history of long-standing pulpitis

- Sclerotic bone – a well-defined radiopacity that is seen below the apices of vital, noncarious teeth

- Hypercemetosis – excess deposition of cementum on root surfaces

- Results from super-eruption, inflammation, or trauma, sometimes no obvious cause

## Anatomical Landmarks

WEBSITES:

## Quizzes on Radiology

## http://www.AndyFutureRDH.com to

## http://healthsci.clayton.edu/bearden/hotpots/

## Radiology Case Studies

http://bpass.dentistry.dal.ca/casestudies.html

UCLA Oral Radiology Online Course

http://www.dent.ucla.edu/sod/depts/oral_rad/courses/DS451c/

Quiz

http://www.dent.ucla.edu/ce/online/case_studies/oral_rad/case001.1.html

Tooth development on panoramics

http://www.uic.edu/classes/orla/orla312/panoramic_x-ray_series.htm

http://w3.dh.nagasaki-u.ac.jp/tf/panorama/index.html

## Dental Radiographic Interpretation Test

http://www.usc.edu/hsc/dental/opath/Guides/BoardQs/SB_01Q.html